# Project KALKI – Phase 1 Developer Guide

## Objective

Build the core engine of a 3D-first AI construction estimation platform where:

- User uploads blueprint PDFs (vector PDFs + scanned drawings)

- System reconstructs a reliable 3D model of the structure

- Material takeoff is generated as a byproduct of validated geometry

- Initial pilot trade = Insulation

- System stores correction feedback using Vector DB (Elastic Memory)

- Output must be future-ready for simulation workflows (thermal / structural)

This is NOT a normal estimator. This is a geometry-first reconstruction engine.

# CORE CONTRACT PILLARS

## Pillar 1 — 3D-First Workflow

Priority order:

PDF → Geometry Extraction → Scale Detection → 3D Reconstruction → Material Logic → Takeoff

Do NOT build 2D-only quantity extraction logic. Takeoff must come from the 3D validated structure.

## Pillar 2 — Simulation-Ready Mesh

3D model must be:

- clean

- watertight

- structured

- exportable

- engineering-compatible

Required export support:

- .OBJ

- .STEP

- OpenUSD (future-ready target)

Future Phase 2 depends on this.

## Pillar 3 — Elastic Memory

System must learn from corrections.

Use Vector Database for:

- storing user corrections

- improving reasoning

- reusable extraction intelligence

- blueprint pattern memory

Suggested:

- FAISS (fast MVP)

- ChromaDB

- Pinecone (future scale)

# PHASE 1 MODULES

# MODULE 1 — PDF INGESTION

## Goal

Accept and normalize uploaded blueprints.

## Input Types

### Vector PDFs

Use:

- PyMuPDF

Extract:

- paths

- lines

- vector geometry

- text blocks

- dimensions

- legends

- notes

- annotations

### Scanned Drawings

Use:

- OpenCV

- OCR if needed

Process:

- grayscale

- thresholding

- denoise

- contour prep

## Deliverable

Normalized blueprint object for pipeline.

# MODULE 2 — GEOMETRY EXTRACTION

## Goal

Understand structure from blueprint.

## Detect

- walls

- room boundaries

- enclosed spaces

- openings

- structural zones

## Vector PDF Strategy

### Logic

- extract line segments

- detect thick parallel wall lines

- remove annotation layers

- graph connected segments

- polygon detection for rooms

## Scanned Drawing Strategy

### OpenCV Pipeline

- adaptive thresholding

- morphology operations

- Canny edge detection

- Hough Transform

- contour detection

- room filtering

## Deliverable

Reliable floor geometry map.

# MODULE 3 — SCALE DETECTION

## Goal

Convert PDF coordinates to real-world dimensions.

## Detection Sources

### Method A

Scale annotation text

Example:

1/4” = 1’-0”

### Method B

Dimension labels

### Method C

Known reference objects

Example:

standard door widths

grid spacing

## Deliverable

Accurate unit conversion engine.

Required:

points/pixels → ft / m / sq ft

# MODULE 4 — PDF TO 3D RECONSTRUCTION ENGINE

## Goal

Generate structured 3D model.

## Requirements

### Build

- floor geometry

- wall heights

- room volumes

- closed surfaces

- zone structure

### Must be

- watertight mesh

- simulation-ready

- exportable

## Suggested Libraries

Possible:

- trimesh

- blender pipeline

- OCC / CAD tools

- IFC support if needed later

## Export Formats

Mandatory:

- OBJ

Preferred:

- STEP

- OpenUSD

## Deliverable

Validated 3D reconstruction engine.

# MODULE 5 — MATERIAL LOGIC (INSULATION PILOT)

## Goal

Use geometry to calculate insulation takeoff.

## NOT simple line counting

Must derive from:

validated wall geometry

## Logic

- identify insulated wall assemblies

- detect exterior walls

- detect cavity walls

- parse insulation notes

- assign insulation specs

- calculate area + volume if needed

## VLM Support

Use GPT-4o or equivalent for:

- legends

- general notes

- material specs

- R-values

- special conditions

## Example

Exterior Wall = R21 Roof = R38

## Deliverable

Structured insulation takeoff.

# MODULE 6 — OUTPUT SCHEMA

## Goal

Deterministic validated outputs.

## Use

Pydantic

## Output Example

{
  "room_id": "R-201",
  "wall_type": "Exterior",
  "insulation_type": "Fiberglass",
  "r_value": "R21",
  "area_sqft": 420,
  "volume": 85,
  "source_reference": "Legend Section B"
}

## Deliverable

Clean production JSON.

# MODULE 7 — VECTOR DB MEMORY

## Goal

System improves from corrections.

## Store

- user corrections

- corrected assumptions

- trade-specific rules

- blueprint patterns

- estimation exceptions

## Example

User says:

“This wall is spray foam not batt insulation”

Store and use later.

## Deliverable

Correction memory system.

# MODULE 8 — DASHBOARD (3D-CENTRIC)

## Goal

3D Render is the center. NOT spreadsheet-first.

## Required Dashboards

### Dashboard A — Project Dashboard

- upload PDFs

- project list

- revision tracking

- processing status

### Dashboard B — 3D Review Dashboard

- rendered 3D model

- room validation

- geometry review

- clickable wall selection

### Dashboard C — Takeoff Dashboard

- quantities

- estimates

- export options

- review status

## Critical Feature

### Interactive Spatial Logic

User clicks 3D wall → system shows:

- reasoning

- material specs

- notes source

- insulation logic

- takeoff explanation

This is mandatory.

# CODE ARCHITECTURE

## Required Structure

project/
│
├── ingestion/
├── geometry/
├── scale/
├── reconstruction/
├── material_logic/
├── schemas/
├── memory/
├── dashboard/
├── exports/
├── pipeline/
└── main.py

## Rules

- modular

- clean

- production quality

- reusable

- no hardcoded hacks

- future-proof architecture

Build for 2030, not 2026.

# PHASE 1 FINAL DELIVERABLES

Developer must deliver:

- PDF ingestion engine

- Geometry extraction engine

- Scale detection system

- PDF → 3D reconstruction engine

- OBJ export support

- Insulation material takeoff engine

- Pydantic validated outputs

- Vector DB correction memory

- 3D-first dashboard structure

- Interactive reasoning system

- Clean modular codebase

- Documentation for Phase 2 expansion

# PHASE 1 SUCCESS CRITERIA

Success means:

User uploads blueprint → System builds reliable 3D structure → User validates geometry → System explains logic → Material takeoff generated → Future trade expansion becomes plug-and-play

That is the true goal.

Not “just another estimator.”